1. Write an assembly language program to add two 16-bit numbers in 8086.

CODE:

MOV BX, [1100H]

MOV CX, [1102H]

ADD BX,CX

MOV [1106H], BX

HLT

INPUT:

[1100H] <- 34

[1101H] <- 12

BX <- 1234

[1102H] <- 78

[1103H] <- 56

CX <- 5678

OUTPUT:

[1106H] <- AC

[1107H] <- 68

BX <- 68AC

2. Write an assembly language program to subtract two 16-bit numbers in 8086.

CODE:

MOV BX, [1100H]

MOV CX, [1102H]

SUB BX,CX

MOV [1106H], BX

HLT

INPUT:

[1100H] <- 34

[1101H] <- 12

BX <- 1234

[1102H] <- 78

[1103H] <- 56

CX <- 5678

OUTPUT:

[1106H] <- BC

[1107H] <- BB

BX <- BBBC

3. Write an assembly language program to multiply two 16-bit numbers in 8086.

CODE:

MOV AX, [1100H]

MOV BX, [1102H]

MUL BX

MOV [1106H], AX

MOV [1108H], DX

HLT

INPUT:

[1100H] <- 11

[1101H] <- 12

AX <- 1211

[1102H] <- 33

[1103H] <- 34

BX <- 3433

OUTPUT:

[1106H] <- 63

[1107H] <- 0D

AX <- 0D63

[1108H] <- AF

[1109H] <- 03

DX <- 03AF

4. Write an assembly language program to divide two 16-bit numbers in 8086.

CODE:

MOV AX, [1100H]

MOV BX, [1102H]

DIV BX

MOV [1106H], AX

MOV [1108H], DX

HLT

INPUT:

[1100H] <- 74

[1101H] <- 02

BX <- 0274 (628 in DEC)

[1102H] <- 19

[1103H] <- 00

CX <- 0019 (25 in DEC)

OUTPUT:

[1106H] <- 19

[1107H] <- 00

AX <- 0019

[1108H] <- 03

[1109H] <- 00

DX <- 0003

5. Write an assembly language program to demonstrate AAA, AAS, AAM, AAD, DAA and DAS in 8086.

AAA:

CODE:

mov AL,'6'

add AL,'7'

aaa

or AL,30H

hlt

OUTPUT:

AL:=36

AL:=36H+37H=6DH

AAA;		AX:=0103H

AL:=33H

AAS:

CODE:

mov AL,'6'

sub AL,'7'

aas

or AL,30H

hlt

OUTPUT:

AL:=36

AL:=36H-37H=FFH

aas;	AX:=FF09H

AX:=FF39H

AAM:

CODE:

mov AL,'3'

mov BL,'9'

and AL,0FH

and BL,0FH

mul BL

aam

or AL,30H

hlt

OUTPUT:

AL:=33H

BL:=39H

AL:=33H AND 0FH = 03H

BL:=39H AND 0FH = 09H

AL:=AL MUL BL = 1BH

aam;	AX:=0207H

AL:= AL OR 30H=37H

AAD:

CODE:

mov AX,0207H

mov BL,05H

aad

div BL

hlt

OUTPUT:

AX:=0207H

BL:=05H

aad;	AX:=001BH

AL DIV BL

AH:=REMAINDER;	AL:=QUOTIENT

DAA:

CODE:

MOV AL, 71H

ADD AL, 43H

DAA

ADD AL, 0AH

DAA

HLT

OUTPUT:

AL <- 71H

AL <- AL+43H

AL<- B4

DAA

MOST SIGNIFICANT 4 bits IN AL>9,

So

AL<- AL+60H;

AL<- 14H ; CY <- 1

AL <- AL+0AH

AL<-1E

DAA

LEAST SIGNIFICANT 4 Bits IN AL>9,

So,

AL <- AL+06H

AL<- 24; AF=1

DAS:

CODE:

MOV AL, 10101011B

SUB AL, 03H

DAS

SUB AL, 0AH

DAS

HLT

OUTPUT:

AL<-10101011B (EQUIVALENT TO AB IN HEX)

AL <- AL - 03H

AL<- A8

DAS

MOST SIGNIFICANT 4 bits OF AL>9,

So

AL <- AL-60H

AL<-48; CF<-1

AL <- AL-0AH

AL <- 3EH

DAS

LEAST SIGNIFICANT 4 bits OF AL>9

SO

AL <- AL-06H

AL<- 38H;AF<-1

6. Write an assembly language program to find out the count of positive numbers and negative numbers

from a series of signed numbers in 8086.

CODE:

MOV CL, 0AH

MOV BL, 00H

MOV DL,00H

LEA SI,[1000H]

L1:MOV AL,[SI]

SHL AL,01

JNC L2

INC DL

JMP L3

L2: INC BL

L3: INC SI

HLT

OUTPUT:

INPUT:

1000:=01

1001:=02

1002:=03

1003:=04

1004:=80

1005:=81

1006:=82

1007:=83

1008:=84

1009:=85

OUTPUT:

CX:=0AH

BL:=NUMBER OF POSITIVE NUMBERS=4

DL:=NUMBER OF NEGATIVE NUMBERS=6

7. Write an assembly language program to convert to find out the largest number from a given unordered

array of 8-bit numbers, stored in the locations starting from a known address in 8086.

CODE:

MOV CL, 0AH

LEA SI,[1000H]

MOV AL,[SI]

L1:INC SI

MOV BL,[SI]

CMP AL,BL

JC L2

JMP L3

L2: MOV AL,BL

L3: DEC CL

JNZ L1

MOV [100AH],AL

HLT

INPUT:

1000:=00H

1001:=01H

1002:=40H

1003:=50H

1004:=33H

1005:=22H

1006:=90H

1007:=09H

1008:=80H

1009:=00H

OUTPUT:

AL:=90 (LARGEST NUMBER IN THAT ARRAY)

8. Write an assembly language program to convert to find out the largest number from a given unordered

array of 16-bit numbers, stored in the locations starting from a known address in 8086.

CODE:

MOV BX, 1000H

MOV CL, [BX]

INC BX

MOV AX, [BX]

DEC CL

BACK: INC BX

INC BX

CMP AX,[BX]

JNC Next

MOV AX, [BX]

Next: DEC CL

JNZ Back

MOV [1020H],AX

HLT

INPUT:

1000:=05H;		CX:=05H

1001:=20H

1002:=30H;		3020H

1003:=40H

1004:=50H;		5040H

1005:=60H

1006:=70H;		7060H

1007:=80H

1008:=90H;		9080H

1009:=10H

100A:=20H;		2010H

OUTPUT:

AX:=9080H (LARGEST 16-BIT NUMBER)

9. Write an assembly language program to print Fibonacci series in 8086.

CODE:

MOV SI,3000H

MOV CX,0AH

XOR AL,AL

MOV [SI],0AH

INC SI

MOV [SI],00H

ADD AL,01H

INC SI

MOV [SI],AL

INC SI

MOV [SI],AL

Back: ADD AL,[SI]

INC SI

MOV [SI],AL

DEC SI

MOV AL,[SI]

INC SI

LOOP Back

HLT

OUTPUT:

3000:=0AH (NUMBER OF FIBONACCI NUMBERS)

10. Write an assembly language program to perform the division 15/6 using the ASCII CODEs. Store the

ASCII CODEs of the result in register DX.

CODE:

MOV AX, '15'

MOV BX, '6'

SUB AX, 3030H

SUB BX, 30H

AAD

DIV BL

ADD AX, 3030H

MOV DX, AX

HLT

OUTPUT:

AX <- ‘15’ => AH <- 31, AL<- 35 (ASCII REPRESENTATION)

BX <- ‘6’ => BX <- 00, AL<- 36 (ASCII REPRESENTATION)

SUB AX, 3030H (TO MAKE AX 0105H)

SIMILARLY, FOR BX

SUB BX, 30H

AAD

AL <- (AH*10)+AL (DECIMAL)  =>  AL <- (01*10)+05=10+5=15=0FH

AH <- 00H

DIV BL

15/6 GIVES 2 QUOTIENT AND 3 REMAINDER IN DECIMAL.

AL <- 02, AH <- 03

ADD AX, 3030H (TO MAKE IT BACK TO ASCII)

STORE AX IN DX.